Presentation:

Person 1: Naman  Bharsakale

[Slide 1 – Title Slide] (0:00 – 0:20)

“Good morning mam.
So, today we are going to present our major project presentation which is ‘Smart Stick for Blind Person’, which aims to improve the mobility and safety of visually impaired individuals.

(Click to next slide)

[Slide 2 – Overview] (0:20 – 0:40)

“In this presentation, 
We’ll begin with an introduction to the idea, followed by the components used, and then explain the working with circuit diagram.
After that, we’ll cover the advantages, limitations, and finally, the future scope of our innovation.”

(Click to next slide)

[Slide 3 – Introduction] (0:40 – 1:30)

“In today’s world, technology can play a vital role in improving the quality of life.
One such innovation is our Smart Stick for Blind Persons, specifically designed to enhance their safety, mobility, and independence.

This stick uses a combination of sensors and microcontrollers to detect obstacles and alert the user in real-time.
We’ve also integrated IoT technology to send emergency alerts in case of any danger.

The core of our system includes an ESP32 microcontroller, ultrasonic sensor, flame sensor, and a buzzer for alerts.
All of this comes together in a compact, assistive device for visually impaired individuals.”

Person 2: Yash Chaukhande

[Slide 4 – Components Used] (0:00 – 0:15)
“In this part, I’ll explain the key components used in our Smart Stick and their roles.
Each of these components was selected to enhance the functionality and safety features of the device.”

[Slide 5 – Components Continued] (0:15 – 1:00)

- ESP32 Microcontroller:
This is the brain of our device. It receives sensor inputs and controls the alert systems accordingly.

- Ultrasonic Sensor:
This detects obstacles in front of the stick by measuring distance using sound waves. It helps prevent collisions.

- Flame Sensor:
It detects the presence of fire or high temperature, alerting the user to potential hazards.

- Buzzer:
Generates sound alerts when an obstacle or fire is detected.

- Vibration Motor:
Gives physical vibration feedback to the user, especially useful in noisy environments.

- Tactile Push Button:
This is an emergency button that the user can press to send alerts.

- Pushbullet API:
Allows the device to send real-time emergency messages to a registered contact.

- Battery/Power Supply:
Powers the entire system and ensures portability.

Person 3: Jay Ganorkar

“This is the complete wiring of our Smart Stick system. Let me explain the connections one by one:

- At the center is the ESP32 microcontroller which controls all components.

- The ultrasonic sensor (HC-SR04) has 4 pins:

  - VCC to 3.3V of ESP32

  - GND to GND

  - TRIG to GPIO pin 27

  - ECHO to GPIO pin 26

- The flame sensor is connected as follows:

  - VCC to 3.3V

  - GND to GND

  - DO (digital output) to GPIO pin 32

- The buzzer is connected between GPIO 19 and GND via a resistor.

- The vibration motor is powered by the main power line, and its signal line is controlled by GPIO pin 18 using a transistor for switching.

- A GPS module is also included:

  - VCC to 3.3V

  - GND to GND

  - TX to RX pin 35

  - RX to TX pin 34

- The push button switch is wired to give digital input to the ESP32 (possibly on GPIO 33, though not labeled), allowing emergency message activation via the Pushbullet API.

- Finally, the power supply is connected through a toggle switch to control the entire circuit.

[Slide 7 – Working] (0:45 – 1:30)
“Now, let me explain the working process of the stick.

When the system is powered on, the ultrasonic sensor starts scanning for obstacles in front of the stick.
If an obstacle is detected within a certain distance — for example, less than 50 cm — the ESP32 triggers both the buzzer and vibration motor, alerting the user instantly.

Simultaneously, the flame sensor keeps monitoring for fire or heat sources. If a flame is detected, the stick again gives a buzzer alert.

In case of any emergency, the user can press the tactile push button, which activates the Pushbullet API.
This sends a message containing the emergency alert to a predefined smartphone or device.

All these features work together to make the stick smart, responsive, and safe for visually impaired users.”

Person 4:  Md Shehzad

[Slide 8 – Advantages] (0:00 – 0:55)
“Our Smart Stick comes with several practical advantages that truly make a difference:

- It helps in real-time obstacle detection using the ultrasonic sensor, alerting the user well in advance.

- The vibration motor gives tactile feedback, which is especially helpful not only for the visually impaired but also for deaf-blind users, or even in noisy environments where a buzzer may go unnoticed.

- The flame sensor adds a safety layer by detecting nearby fire sources.

- The emergency push button sends alerts via the Pushbullet API, keeping the user connected in case of danger.

- One of the biggest benefits is the affordable cost – all the components are low-cost and easily available.

- The battery backup is efficient — it can run continuously for 6 hours, and on regular use, it lasts for 2 to 3 days without charging.

This makes the smart stick practical, reliable, and user-friendly for daily mobility and safety.”

(Transition to next slide)

[Slide 9 – Limitations] (0:55 – 1:30)
“Now, let’s look at a few limitations:

- The flame sensor can sometimes give false alerts. For example, in high temperature environments, it may falsely detect flame and cause continuous buzzing.

- The emergency alert feature depends on Wi-Fi. If the stick isn’t connected to a stable internet connection, the Pushbullet API can’t send notifications.

- And lastly, the ultrasonic sensor has a limited detection range — it may miss fast-moving or very small objects.

Despite these minor challenges, our project delivers real value, and we already have ideas to improve it further.”

Person 5 : Sarvesh Chaudhari

[Slide 11 – Future Scope (Elaborated)] (0:35 – 1:25)
“For future improvements, we’ve identified a few impactful directions:

- AI-Based Object Recognition:
By integrating a small camera module and AI algorithms, the stick could identify and recognize objects — like doors, stairs, or vehicles — and alert the user with detailed feedback. This will help in making better navigation decisions, beyond just distance sensing.

- Multilingual Voice Feedback:
We plan to add voice-based alerts in multiple languages, such as Hindi, Marathi, or English. This would make the stick more inclusive and accessible to users from different regions, especially in India, where language diversity is high.

- Solar Charging System:
To support portability and sustainability, a small solar panel module can be added to recharge the battery during the day. This would make the stick eco-friendly and usable in remote areas with limited access to electricity.

These enhancements will transform the stick into a more intelligent, user-friendly, and self-sustaining device.”

(Transition to Slide 12)

[Slide 12 – Conclusion (Updated)] (1:25 – 1:45)
“To conclude:

The IoT-Based Smart Stick for Blind Persons is a reliable, affordable, and innovative assistive device that improves mobility and safety for visually impaired individuals.
By combining sensors, microcontrollers, and IoT features, the stick helps users detect obstacles, avoid fire hazards, and send emergency alerts.

This project shows how simple technology can create a powerful social impact — and opens doors to future innovations like AI integration, solar power, and multilingual voice support.

It’s a meaningful step toward building a more inclusive and accessible world through smart engineering solutions.

Thank you.”